| Modbus Register Type | PLC Address | Offset (0-based) | Tag Name | Data Type | Range / Values | Functional Description |
| --- | --- | --- | --- | --- | --- | --- |
| Coil (Read/Write) | 00001 | 0 | SYS_RUN_STOP | Boolean | 0 = STOP, 1 = START | Production line master start/stop from QModBus / SCADA |
| Discrete Input (RO) | 10001 | 0 | SENSOR_PIECE_DET | Boolean | 0 = Absent, 1 = Present | Trigger active when HC-SR04 detects a workpiece (< 10 cm) |
| Discrete Input (RO) | 10002 | 1 | QC_STATUS | Boolean | 0 = Defect, 1 = OK | Optical and thermal inspection result for the inspected piece |
| Input Register (RO) | 30001 | 0 | DIST_HC_SR04 | uint16 | 2 - 400 cm | Real-time distance measurement from ultrasonic sensor |
| Input Register (RO) | 30002 | 1 | LDR_INTENSITY | uint16 | 0 - 4095 | Raw ADC intensity value for optical surface / color check |
| Input Register (RO) | 30003 | 2 | DHT11_TEMP | int16 | 0 - 50 °C | Ambient environment temperature from DHT11 |
| Input Register (RO) | 30004 | 3 | DHT11_HUM | uint16 | 20 - 90 %RH | Relative humidity of the working environment |
| Input Register (RO) | 30005 | 4 | TOTAL_OK_PIECES | uint16 | 0 - 65535 | Cumulative counter of verified non-defective parts |
| Input Register (RO) | 30006 | 5 | TOTAL_REJECTS | uint16 | 0 - 65535 | Cumulative counter of rejected and sorted scrap parts |
| Holding Register (RW) | 40001 | 0 | OPERATION_MODE | uint16 | 0 = Manual, 1 = Auto | System operating mode (Manual testing vs. Full automatic) |
| Holding Register (RW) | 40002 | 1 | TEMP_THRESHOLD | uint16 | Default: 25 °C | High temperature cutoff threshold triggering reject state |
| Holding Register (RW) | 40003 | 2 | LDR_THRESHOLD | uint16 | 0 - 4095 | Optical decision cutoff separating accepted vs. rejected parts |
| Holding Register (RW) | 40004 | 3 | MANUAL_SERVO_ANGLE | uint16 | 0 - 180° | Direct servo angle positioning command (Manual mode only) |
| Holding Register (RW) | 40005 | 4 | STEPPER_MANUAL_STEPS | uint16 | 0 - 4096 steps | Direct conveyor step displacement command for calibration |